| Last Modified: 9-4-2007 |  | 1.6 C |
| --- | --- | --- |
| Service Category: Audio/Visual/Telematics | Section: Park Assist/Monitoring |  |
| Model Year: 2008 | Model: Land Cruiser | Doc ID: RM000003AHC000X |
| Title: PARK ASSIST / MONITORING: INTUITIVE PARKING ASSIST SYSTEM: C1AEC: Front Sensor Communication Malfunction (2008 Land Cruiser) |  |  |

| DTC | C1AEC | Front Sensor Communication Malfunction |
| --- | --- | --- |

## DESCRIPTION

This DTC is stored when there is an open or short circuit in the communication line between the front sensors and the ECU, or when there is a malfunction in a front sensor.

| DTC CODE | DTC DETECTION CONDITION | TROUBLE AREA |
| --- | --- | --- |
| C1AEC | An open or short circuit in the communication line between the front sensors and ECU or a malfunction in a front sensor during initialization mode after the engine switch is turned on (IG). | - Front ultrasonic sensor circuit - Clearance warning ECU |

## WIRING DIAGRAM

## INSPECTION PROCEDURE

## PROCEDURE

| 1. | CHECK DTC OUTPUT |
| --- | --- |

(a) Clear the DTCs  .

(b) Check for DTCs  .

#### Result

| RESULT | PROCEED TO |
| --- | --- |
| DTC C1AEC is output | A |
| No DTC is output | B |

| B |  | USE SIMULATION METHOD TO CHECK |
| --- | --- | --- |

A.

| 2. | CHECK CLEARANCE WARNING ECU |
| --- | --- |

(a) Measure the voltage according to the value(s) in the table below.

Standard Voltage:

| TESTER CONNECTION | CONDITION | SPECIFIED CONDITION |
| --- | --- | --- |
| F12-21 (BOF) - Body ground | Engine switch on (IG) | 7.2 to 8.8 V |

(b) Measure the resistance according to the value(s) in the table below.

Standard Resistance:

| TESTER CONNECTION | CONDITION | SPECIFIED CONDITION |
| --- | --- | --- |
| F12-19 (E1) - Body ground | Always | Below 1 Ω |

| NG |  | REPLACE CLEARANCE WARNING ECU |
| --- | --- | --- |

OK.

| 3. | CHECK HARNESS AND CONNECTOR (NO. 1 ULTRASONIC SENSOR LH - CLEARANCE WARNING ECU) |
| --- | --- |

(a) Disconnect the B4 sensor connector.

(b) Disconnect the F12 ECU connector.

(c) Measure the resistance according to the value(s) in the table below.

Standard Resistance:

| TESTER CONNECTION | CONDITION | SPECIFIED CONDITION |
| --- | --- | --- |
| B4-1 (BI) - F12-21 (BOF) | Always | Below 1 Ω |
| B4-3 (SI) - F12-9 (SOF) | Always | Below 1 Ω |
| B4-5 (EI) - F12-19 (EI) | Always | Below 1 Ω |
| B4-1 (BI) - Body ground | Always | 10 kΩ or higher |
| B4-3 (SI) - Body ground | Always | 10 kΩ or higher |
| B4-5 (EI) - Body ground | Always | 10 kΩ or higher |

| NG |  | REPAIR OR REPLACE HARNESS OR CONNECTOR |
| --- | --- | --- |

OK.

| 4. | CHECK HARNESS AND CONNECTOR (NO. 1 ULTRASONIC SENSOR LH - NO. 1 ULTRASONIC SENSOR RH) |
| --- | --- |

(a) Disconnect the B4 and B3 sensor connectors.

(b) Measure the resistance according to the value(s) in the table below.

Standard Resistance:

| TESTER CONNECTION | CONDITION | SPECIFIED CONDITION |
| --- | --- | --- |
| B4-2 (BO) - B3-1 (BI) | Always | Below 1 Ω |
| B4-4 (SO) - B3-3 (SI) | Always | Below 1 Ω |
| B4-6 (EO) - B3-5 (EI) | Always | Below 1 Ω |
| B4-2 (BO) - Body ground | Always | 10 kΩ or higher |
| B4-4 (SO) - Body ground | Always | 10 kΩ or higher |
| B4-6 (EO) - Body ground | Always | 10 kΩ or higher |

| NG |  | REPAIR OR REPLACE HARNESS OR CONNECTOR |
| --- | --- | --- |

OK.

| 5. | CHECK DTC OUTPUT |
| --- | --- |

(a) Clear the DTCs  .

(b) Check for DTCs  .

#### Result

| RESULT | PROCEED TO |
| --- | --- |
| DTC C1AEC is output and multi-information display indicates open circuit (Front Left and Front Right) | A |
| No DTC is output | B |

| B |  | USE SIMULATION METHOD TO CHECK |
| --- | --- | --- |

A.

| 6. | REPLACE NO. 1 ULTRASONIC SENSOR LH |
| --- | --- |

(a) Replace the No. 1 ultrasonic sensor LH with a normally functioning sensor  .

NEXT.

| 7. | CHECK DTC OUTPUT |
| --- | --- |

(a) Clear the DTCs  .

(b) Check for DTCs  .

#### Result

| RESULT | PROCEED TO |
| --- | --- |
| DTC C1AEC is output | A |
| No DTC is output | B |

| B |  | END (NO. 1 ULTRASONIC SENSOR LH IS DEFECTIVE) |
| --- | --- | --- |

A.

| 8. | REPLACE NO. 1 ULTRASONIC SENSOR RH |
| --- | --- |

(a) Replace the No. 1 ultrasonic sensor RH with a normally functioning sensor  .

NEXT.

| 9. | CHECK DTC OUTPUT |
| --- | --- |

(a) Clear the DTCs  .

(b) Check for DTCs  .

#### Result

| RESULT | PROCEED TO |
| --- | --- |
| DTC C1AEC is output | A |
| No DTC is output | B |

| B |  | END (NO. 1 ULTRASONIC SENSOR RH IS DEFECTIVE) |
| --- | --- | --- |

| A |  | REPLACE CLEARANCE WARNING ECU |
| --- | --- | --- |